# Bill of Materials (BOM)

## Device

- NVIDIA Jetson Nano DEVELOPER KIT
GPU acceleration for YOLO model
required to reach 15~20 FPS
No need to use pigpio, just Jetson.GPIO is ok

## Camera

- Logitech C920 HD Pro Webcam
USB plug-and-play 
stable working with OpenCV

## Servo Controller

- Adafruit PCA9685 16-Channel PWM/Servo Driver
Provides stable PWM via I2C
avoids jitter from direct GPIO control

## Pan-Tilt

- SunFounder Pan Tilt Kit
Simple 2-axis mount for camera and laser

## Servos (x2)

- MG996R High Torque Servo
Higher torque and stability compared to micro servos
suitable for mounted load.

## Laser Module

- 5W 450nm TTL Laser Module

## Power Supply

- 5V 4A Power Supply for Jetson Nano

- 6V External Battery Pack for Servos
 Isolates servo power to prevent voltage drops and instability.

## Other Parts Needed

- Jumper wires (M/F, F/F)

- Breadboard or terminal block

## Optional (Recommended)

- Physical kill switch for laser

- Basic enclosure for outdoor protection(rain, snow, wind, etc)